Quad_Motor_Driver_Shield_for_Arduino_SKU_DRI0039-DFRobot

https://www.dfrobot.com/product-1513.html

## Introduction

The latest quad motor driver shield for Arduino, compatible with 5V/3.3V Arduino controller, can control up to four DC motors with 8 pins at the same time. The shield supports PWM speed control and polarity control. The driver shield includes two TB6612FNG motor driver chips, compared with the traditional L298N chip, efficiency is improved and the component size also greatly reduced. The chip doesn't heat in to the rated range, and a single path maximum output 1.2A continuous current. The module includes a built-in low voltage detection circuit and thermal shutdown protection circuit, which is safe and reliable. This module is suitable for all kinds of DIY production.

## Specification

- VM Motor Power Supply: 2.5 V ~ 13.5 V

- VCC Logic Power Supply: 2.7 ~ 5.5 V

- Output Current: 1.2A continuous current (per channel)

- Start/Peak Current: 2A (continuous pulse) / 3.2A (single pulse)

- Arduino Control Port: Digital pins 3, 4, 5, 6, 7, 8, 11, 12

- Working Temperature: -20 ~ 85 ℃

- Dimensions: 53mm x 52mm/2.09 x 2.05

## Board Overview

| Motor | Direction | Forward | Backward | Speed | Speed range |
| --- | --- | --- | --- | --- | --- |
| M1 | 4 | LOW | HIGH | 3 | 0-255 |
| M2 | 12 | HIGH | LOW | 11 | 0-255 |
| M3 | 8 | LOW | HIGH | 5 | 0-255 |
| M4 | 7 | HIGH | LOW | 6 | 0-255 |

Motor I/O

## Tutorial

### You Will Need

- Hardware

  - DFRduino UNO R3 x 1

  - Quad Motor Shield for Arduino x 1

  - DC motor x 4

  - Jumper Wires

- Software

  - Arduino IDE Click to download Arduino IDE

### Connection Diagram

### Sample Code

/*!

* @file QuadMotorDriverShield.ino

* @brief QuadMotorDriverShield.ino Motor control program

* Every 2 seconds to control motor positive inversion

* @author linfeng(490289303@qq.com)

* @version V1.0

* @date 2016-4-5

*/

const int E1 =3;///<Motor1 Speed

const int E2 =11;///<Motor2 Speed

const int E3 =5;///<Motor3 Speed

const int E4 =6;///<Motor4 Speed

const int M1 =4;///<Motor1 Direction

const int M2 =12;///<Motor2 Direction

const int M3 =8;///<Motor3 Direction

const int M4 =7;///<Motor4 Direction

voidM1_advance(char Speed)///<Motor1 Advance

{

digitalWrite(M1,LOW);

analogWrite(E1,Speed);

}

voidM2_advance(char Speed)///<Motor2 Advance

{

digitalWrite(M2,HIGH);

analogWrite(E2,Speed);

}

voidM3_advance(char Speed)///<Motor3 Advance

{

digitalWrite(M3,LOW);

analogWrite(E3,Speed);

}

voidM4_advance(char Speed)///<Motor4 Advance

{

digitalWrite(M4,HIGH);

analogWrite(E4,Speed);

}

voidM1_back(char Speed)///<Motor1 Back off

{

digitalWrite(M1,HIGH);

analogWrite(E1,Speed);

}

voidM2_back(char Speed)///<Motor2 Back off

{

digitalWrite(M2,LOW);

analogWrite(E2,Speed);

}

voidM3_back(char Speed)///<Motor3 Back off

{

digitalWrite(M3,HIGH);

analogWrite(E3,Speed);

}

voidM4_back(char Speed)///<Motor4 Back off

{

digitalWrite(M4,LOW);

analogWrite(E4,Speed);

}

voidsetup(){

for(int i=3;i<9;i++)

pinMode(i,OUTPUT);

for(int i=11;i<13;i++)

pinMode(i,OUTPUT);

}

voidloop(){

M1_advance(100);

M2_advance(100);

M3_advance(100);

M4_advance(100);

delay(2000);///<Delay 2S

M1_back(100);

M2_back(100);

M3_back(100);

M4_back(100);

delay(2000);///<Delay 2S

}

### Result

You should see the motor go forwards and backwards every two seconds

## FAQ

For more questions or interesting projects, you can Visit the forum!

## More

Schematic

Layout

TB6612 Datasheet

SVG files

Shopping from Quad Motor Driver Shield for Arduino or DFRobot Distributor.

Turn to the Top